OBJECTIVE 1 & 2:

clear; clc; close all;

% Illustrative system parameters

totalBandwidthHz = 1e6; % Total bandwidth: 1 MHz

fdmaChannelHz = 200e3; % FDMA channel width: 200 kHz

tdmaSlots = 8; % Number of TDMA slots per frame

processingGain = 64; % CDMA processing gain

targetSIRdB = 7; % Minimum acceptable CDMA SIR

% Capacity estimates under the stated assumptions

fdmaUsers = floor(totalBandwidthHz / fdmaChannelHz);

tdmaUsers = tdmaSlots;

targetSIR = 10^(targetSIRdB / 10);

cdmaUsers = floor(1 + processingGain / targetSIR);

fprintf('Illustrative maximum supported users\n');

fprintf('FDMA: %d users\n', fdmaUsers);

fprintf('TDMA: %d users\n', tdmaUsers);

fprintf('CDMA: %d users at a target SIR of %.1f dB\n\n', ...

cdmaUsers, targetSIRdB);

% CDMA SIR as the number of simultaneous users increases

K = 2:30;

sirLinear = processingGain ./ (K - 1);

sirDb = 10 * log10(sirLinear);

% Display a comparison table

method = ["FDMA"; "TDMA"; "CDMA"];

supportedUsers = [fdmaUsers; tdmaUsers; cdmaUsers];

results = table(method, supportedUsers);

disp(results);

% Plot estimated user capacity

figure;

bar(categorical(method), supportedUsers);

ylabel('Illustrative maximum supported simultaneous users');

title('FDMA, TDMA and CDMA: Capacity Estimate');

grid on;

% Plot estimated CDMA SIR as users are added

figure;

plot(K, sirDb, 'o-', 'LineWidth', 1.5);

hold on;

yline(targetSIRdB, '--r', 'Target SIR');

xlabel('Number of simultaneous CDMA users (K)');

ylabel('Estimated SIR (dB)');

title('Estimated CDMA SIR versus Number of Users');

grid on;

hold off;

OBJECTIVE 3:

generation = ["1G"; "2G"; "2G"; "3G"; "4G"; "5G"];
system = ["AMPS"; "GSM"; "IS-95"; "UMTS"; "LTE"; "5G NR"];
accessMethod = ["FDMA"; "TDMA"; "CDMA"; "WCDMA"; "OFDMA"; "OFDMA-based"];
keyIdea = ["Separate frequency channels"; ...
"Separate time slots"; ...
"Code-based user separation"; ...
"Wideband code-based access"; ...
"Orthogonal frequency resources"; ...
"Flexible OFDM-based radio access"];

evolutionTable = table(generation, system, accessMethod, keyIdea);
disp(evolutionTable);

OBJECTIVE 4:

figure;
tiledlayout(1,3);
% FDMA: users occupy different frequency channels
nexttile;
hold on;
colors = lines(4);
for u = 1:4
rectangle('Position',[0, u-1, 1, 0.8], ...
'FaceColor',colors(u,:), 'EdgeColor','k');
text(0.5, u-0.6, sprintf('User %d',u), ...
'HorizontalAlignment','center');
end
xlim([0 1]); ylim([0 4]);
xlabel('Time');
ylabel('Frequency channel');
title('FDMA');

set(gca,'YTick',[]);
hold off;
% TDMA: users occupy different time slots on one frequency channel
nexttile;
hold on;
for u = 1:4
rectangle('Position',[u-1, 0, 0.8, 1], ...
'FaceColor',colors(u,:), 'EdgeColor','k');
text(u-0.6, 0.5, sprintf('User %d',u), ...
'HorizontalAlignment','center');
end
xlim([0 4]); ylim([0 1]);
xlabel('Time slot');
ylabel('Frequency channel');
title('TDMA');
set(gca,'YTick',[]);
hold off;
% CDMA: users share the same time and frequency band using different codes
nexttile;
hold on;
for u = 1:4
rectangle('Position',[0, 0, 1, 1], ...
'FaceColor',colors(u,:), 'FaceAlpha',0.22, ...
'EdgeColor',colors(u,:), 'LineWidth',1.5);
end
text(0.5, 0.62, 'Users 1-4 share', 'HorizontalAlignment','center');
text(0.5, 0.40, 'band; different codes', 'HorizontalAlignment','center');
xlim([0 1]); ylim([0 1]);
xlabel('Time');
ylabel('Frequency band');
title('CDMA');
set(gca,'YTick',[]);
hold off;
sgtitle('Conceptual Time-Frequency Resource Allocation');